| 1. Part Number |  |  |  | 2. Part Name |  |  |  | 3. Serial Number | 4. FAI Report |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12345 REV.AF |  |  |  | BALLSCREW |  |  |  | 001 | T-7446-1 |
| Characteristic Accountability |  |  |  | Inspection / Test Results |  |  |  | Optional Fields |  |
| 5. Char No. | 6. Ref. Location | 7. Char. Designator | 8. Requirement | 9. Results | 10. Designed Tooling |  | 11. Non-Conformance Number | 14. REMARKS |  |
| 1 | D7 |  | PERP W/IN .001”/C |  |  |  |  | Cannot check - ECO pending. |  |
| 2 | D7 |  | .0132” +/-.0003 | .0129 |  |  |  |  |  |
| 3 | D7 |  | .0066 +/-.0003 | .0064 |  |  |  |  |  |
| 4 | D7 |  | 16 MICROFINISH | 16 |  |  |  |  |  |
| 5 | C8 |  | R .0875” +/-.0005 | .0875 |  |  |  |  |  |
| 6 | C7 |  | R .0875” +/-.0005 | .0875 |  |  |  |  |  |
| 7 | D6 |  | .0066” +/-.0003 | .0067 |  |  |  |  |  |
| 8 | C8 |  | Ø .865” +0/-.001 | .8643 |  |  |  | SEE ALSO B/P NOTE 3 |  |
| 9 | C8 |  | Ø .500” +.003/-0 | .502 / .503 |  |  |  |  |  |
| 10 | B8 |  | Ø .700” +0/-.002 | .6993 |  |  |  |  |  |
| 11 | B8 |  | < 60DEG +/-1DEG | 60 DEG |  |  |  |  |  |
| 12 | B8 |  | Ø .56” +/-.01 | .565 |  |  |  |  |  |
| 13 | B7 |  | .130” +.005/-0 | .1325 |  |  |  |  |  |
| 14 | B6 |  | 14.028” +/-.005 | 14.0247 |  |  |  |  |  |
| 15 | B5 |  | <45DEG +/-1DEG | 45 DEG |  |  |  |  |  |
| 16 | B5 |  | .02” +/-.01 | .03 |  |  |  |  |  |
| 17 | B4 |  | 3.10” +/-.01 | 3.099 |  |  |  |  |  |
| 18 | B3 |  | Ø .9370” +0/-.0003 | .9368 |  |  |  |  |  |
| 19 | B3 |  | TR POS W/IN .005”/A | W/IN .0015 |  |  |  |  |  |
| 20 | B3 |  | .640” +0/-.002 | .6391 |  |  |  |  |  |
| 21 | C3 |  | 1.280” +0/-.004 | 1.2784 |  |  |  |  |  |
| 22 | C3 |  | SMOOTH BLEND | YES |  |  |  |  |  |
| 23 | C3 |  | R .639”+/-.002 | .6385 |  |  |  |  |  |
| 24 | C4 |  | 2.639” +0/-.005 | 2.6384 |  |  |  |  |  |
| 25 | C4 |  | PERP W/IN .001”/B | W/IN .0002 |  |  |  | ECO adds MMC |  |
| 26 | C5 |  | R .38” +/-.01 | .378 |  |  |  |  |  |
| 1. Part Number |  |  |  | 2. Part Name |  |  |  | 3. Serial Number | 4. FAI Report |
| 12345 REV.AF |  |  |  | BALLSCREW |  |  |  | 001 | T-7446-1 |
| Characteristic Accountability |  |  |  | Inspection / Test Results |  |  |  | Optional Fields |  |
| 5. Char No. | 6. Ref. Location | 7. Char. Designator | 8. Requirement | 9. Results | 10. Designed Tooling |  | 11. Non-Conformance Number | 14. REMARKS |  |
| 27 | C6 |  | Ø 1.0313” +0/-.0006 | 1.0310 |  |  |  | OVER .15625 PINS |  |
| 28 | C6 |  | RUNOUT W/IN.001”/A | W/IN .0004 |  |  |  |  |  |
| 29 | D5 |  | Ø 1.0000” +.0008/-0 | 1.0005 |  |  |  |  |  |
| 30 | D5 |  | TR POS W/IN .002”/B | W/IN .0005 |  |  |  | ECO adds MMC |  |
| 31 | D5 |  | Ø 1.278” +0/-.002 | 1.2771 |  |  |  |  |  |
| 32 | C4 |  | .09” +/-.01 | .08 |  |  |  |  |  |
| 33 | C4 |  | 2.000” +/-.005 | 2.0004 |  |  |  |  |  |
| 34 | C4 |  | <7DEG +/-1DEG | 7 DEG |  |  |  |  |  |
| 35 | C3 |  | .221” +/-.001 | .2206 |  |  |  |  |  |
| 36 | C3 |  | .442” +0/-.002 | .4411 |  |  |  |  |  |
| 37 | D4 |  | .83” +/-.01 | .833 |  |  |  |  |  |
| 38 | D4 (2X) |  | R .50” +/-.01 | .500 |  |  |  |  |  |
| 39 | D4 |  | R .740” +.005/-0 | .7445 |  |  |  |  |  |
| 40 | D4 |  | .740” +.005/-0 | .7432 |  |  |  |  |  |
| 41 | B2 |  | <10DEG +/-1DEG | 10 DEG |  |  |  |  |  |
| 42 | B3 (4X) |  | R .32” +/-.01 | .320 |  |  |  |  |  |
| 43 | A4 |  | RUNOUT TIR W/IN .005” | W/IN .005 |  |  |  |  |  |
| 44 | A4 |  | 125 MICROFINISH | 125 |  |  |  |  |  |
| 45 | A4 |  | R .015” +/-.005 | .012 / .018 |  |  |  |  |  |
| 46 | A4 |  | BREAK SHARP EDGES | YES |  |  |  |  |  |
| 47 | A3 |  | MAT’L 9310 AMS6260 | CERTS |  |  |  | TITLE BLOCK |  |
| 48 | A8, A6 |  | CARBURIZE | CERTS |  |  |  | B/P NOTE 2, 3 & 12 |  |
| 49 | A8 |  | MAG PART INSP | CERTS |  |  |  | B/P NOTE 1 |  |
| 50 | B8 |  | CAD PLATE | CERTS |  |  |  | B/P NOTE 7 |  |
| 51 | B8 |  | STRESS RELIEVE | CERTS |  |  |  | B/P NOTE 7.2 |  |
| 52 | B8 |  | HYD EMBR RELIEVE | CERTS |  |  |  | B/P NOTE 7.3 |  |
| 1. Part Number |  |  |  | 2. Part Name |  |  |  | 3. Serial Number | 4. FAI Report |
| 12345 REV.AF |  |  |  | BALLSCREW |  |  |  | 001 | T-7446-1 |
| Characteristic Accountability |  |  |  | Inspection / Test Results |  |  |  | Optional Fields |  |
| 5. Char No. | 6. Ref. Location | 7. Char. Designator | 8. Requirement | 9. Results | 10. Designed Tooling |  | 11. Non-Conformance Number | 14. REMARKS |  |
| 53 | A8 |  | COLD STABILIZATION | CERTS |  |  |  | B/P NOTE 6 |  |
| 54 | A8, A6 |  | POST GRND SAMPLE | CERTS |  |  |  | B/P NOTE 5 |  |
| 55 | A6 |  | ID, OIL, PKG | CONFORMS |  |  |  | B/P NOTE 8 & 11 |  |
| The signature indicates that all characteristics are accounted for; meet drawing requirements or are properly documented for disposition. |  |  |  |  |  |  |  |  |  |
| 12. Prepared By |  |  |  |  |  | 13. Date |  |  |  |